3. 구성도

GSR12416

GSR12416

Black

Diamond

Black

Diamond

g1/0

g2/0

GSR12410

남수원

6509

용인

6509

남수원 g1/1 220.xxx.xxx.105/30

수원   g1/2 220.xxx.xxx.109/30

신갈   g2/1 220.xxx.xxx.113/30

용인   g2/2 220.xxx.xxx.117/30

g0/0 220.xxx.xxx.106/30

g0/0 220.xxx.xxx.118/30

수원

6509

신갈

6509

g0/0 220.xx.xxx.110/30

g0/0 220.xxx.xxx.114/30

4. 주요 작업 내역

| 일차 | 주요작업내용 | 예정시간 | 장애시 대책 | 확인 |
| --- | --- | --- | --- | --- |
| 1 | 1. 사전작업 1.1 해당 국소 작업 공지 1.2 Routing 정보 확인 1.3 Interface traffic 확인 | 20분 |  |  |
|  | 2. 수원 중계라우터 GSR12416 연동 2.1 물리적 Link 연결 2.2 GSR#1,2 IGP(OSPF) 라우팅 등록 2.3 GSR#1,2 BGP 라우팅 등록 2.4 MPLS 정보 등록 2.5 Routing 정보 확인 2.6 Port별 트래픽 확인 | 40분 |  |  |
|  | 3. KT-VPN M/E 스위치 CAT6509 연동 3.1 물리적 Link 연결 확인 3.2 Static Routing 정보 설정 3.3 Routing 정보 확인 3.4 Port별 트래픽 확인 | 20분 |  |  |
|  | 4. 모니터링 및 Configuration 저장 4.1 configuration 저장 및 확인 4.2 모니터링 | 1시간 |  |  |

5. 상세작업내용

5.1 사전작업

5.1.1 해당 국소 작업 공지

5.1.2  Routing 정보 확인

!

show ip int brief

show ip ospf neighbor

show ip ospf database

show ip bgp sum

show ip bgp

show ip cef sum

show ip route

show mpls forwarding-table

show mpls interfaces

show tag-switching forwarding-table

5.1.3  Interface별 트래픽 확인

!

show fdb

show ports utilization

top

show vlan detail

5.2. 수원 중계라우터 GSR12416 연동

- 5. 라우팅 연동 작업

5.1 물리적 구성도

(혜화, 구로, 영동, 인천, 남수원)

* 주 : 혜화 국사는 BD대신 임시로 Summit 스위치 사용

5.3 MPLS LDP 설정

상위망과 MPLS 연동을 하기위해 LDP를 설정합니다.

ROUTER(config)#mpls label protocol ldp

!  MPLS 프로토콜을 LDP로 정의합니다

ROUTER(config)#mpls ldp logging neighbor-changes

!  LDP neighbor 의 상황변동을 Logging합니다

ROUTER(config)#Tag-switching advertise-tags for 1

ROUTER(config)#Tag-switching ldp router-id loopback0

! Router-ID를 lookback0 address를 이용/

ROUTER(config)#interface GigabitEthernet0/0

ROUTER(config-if)#tag-switching ip

!  Interface에 MPLS를 enable 시킵니다.

ROUTER(config-if)#tag-switching mtu 1516

!  MPLS지원을 위해 MTU 사이즈를 조정합니다.

기본적인 LDP구성 상태를 확인합니다.

ROUTER#sh mpls ldp nei

Peer LDP Ident: 192.168.128.171:0; Local LDP Ident 192.168.128.170:0

TCP connection: 7.7.7.4.11317 - 7.7.7.2.646

State: Oper; Msgs sent/rcvd: 5463/5462; Downstream

Up time: 06:32:58

LDP discovery sources:

GigabitEthernet0/0, Src IP addr: 220.xxx.xxx.3

Addresses bound to peer LDP Ident:

192.168.128.171    192.168.128.172

5.4 Connection 설정

기존 12416 라우터와 연결을 설정 확인합니다.

- ROUTER(config)#interface GigaEhternet 0/0

- ROUTER(config-if)#ip add 220.xxx.xxx.95 255.255.255.240

(국사에 따라 위에 표에 맞추어 설정)

- ROUTER(config-if)#no shutdown

- ROUTER(config-if)#exit

- ROUTER(config)#interface GigaEhternet 1/0

- ROUTER(config-if)#ip add 220.xxx.xxx.115 255.255.255.240

(국사에 따라 위에 표에 맞추어 설정)

- ROUTER(config-if)#no shutdown

- ROUTER(config-if)#exit

- Ping으로 12416과의 연결 확인

- ROUTER#ping 220.xxx.xxx.97		; to 12416#1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 220.xxx.xxx.97, timeout is 2 seconds:

!!!!!

- Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/4 ms

- ROUTER#ping 220.xxx.xxx.98		; to 12416#2

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 220.xxx.xxx.98, timeout is 2 seconds:

!!!!!

- Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/4 ms

5.5 OSPF 설정

MPLS VPN 망과의 연동을 위해 OSPF 라우팅 프로토콜을 설정합니다.

- ROUTER#conf t

!  Interface를 Config 하기 위하여 Global config Mode로 전환합니다.

- ROUTER(config)#router ospf 100

!  ospf process를 enable합니다. ( 1=process number)

- ROUTER(config-router)#network 220.xxx.xxx.0 0.0.0.15 area 0

!  상단 GSR12416과 OSPF 통신을 할 IP 네트워크에 대하여 OSPF area 0 로 설정

- ROUTER(config-router)#network 192.168.128.173 0.0.0.0 area 0

!  관리목적으로 배정된 자신의 loopback address를 OSPF에 연동

(구로:192.168.128.171, 영동: 192.168.128.172, 혜화:192.168.128.170, 남인천: 192.168.128.174)

- ROUTER(config-router)#network 220.xxx.xxx.0 0.0.0.3 area 0

!  관리목적으로 배정된 하위의 6509스위치의 IP address를 OSPF에 연동

- ROUTER(config-router)#end

ROUTER#wr

- Router#sh ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is 10.1.1.254 to network 0.0.0.0

192.168.128.0/32 is subnetted, 2 subnets

C       192.168.128.170 is directly connected, Loopback0

20.0.0.0/24 is subnetted, 1 subnets

C       20.1.1.0 is directly connected, GigabitEthernet1/0

220.xxx.xxx.0/24 is subnetted, 1 subnets

O       220.xxx.xxx.12 [20/0] via 220.xxx.xxx.1, 00:02:55, GigabitEthernet1/0

-> 	다른 국사에 관리 목적으로 배정된 220.xxx.xxx.0 네트워크가 넘어오면 성공

5.7 BGP 설정

MPLS VPN 망과의 연동을 위해 BGP 라우팅 프로토콜을 설정합니다.

- ROUTER#conf t

!  Interface를 Config 하기 위하여 Global config Mode로 전환합니다.

- ROUTER(config)#router bgp 4766

!  AS number를 4766으로 지정하여 BGP 라우팅을 설정합니다

- ROUTER(config-router)#bgp  router-id 192.168.128.173

ㅣ Loopback address를 router-id로 지정.

- ROUTER(config-router)#bgp log-neighbor-changes

- !  BGP neighbor의 상태가 변화할 때마다 정보가 logging 되도록 설정합니다.

- ROUTER(config-router)#neighbor 192.168.128.66 remote-as 4766

- ROUTER(config-router)#neighbor 192.168.128.66 description # NanInchon-BBR#

- !  수원국사의 라우터를 BGP neighbor로 선언하고 AS 4766으로하여 IBGP를 설정합니다.

- ROUTER(config-router)#neighbor 192.168.128.66 update-source Loopback0

- !  BGP neighbor에게 자신을 알릴 때, Loopback0 를 이용하도록 설정합니다

neighbor 192.168.128.3 remote-as 4766

neighbor 192.168.128.3 description # VPN_GSR1 #

neighbor 192.168.128.3 shutdown

neighbor 192.168.128.3 update-source Loopback0

neighbor 192.168.128.4 remote-as 4766

neighbor 192.168.128.4 description # VPN_GSR2 #

neighbor 192.168.128.4 shutdown

neighbor 192.168.128.4 update-source Loopback0

neighbor 192.168.128.235 remote-as 4766

neighbor 192.168.128.235 description # internet-pe8 #

neighbor 192.168.128.235 update-source Loopback0

neighbor 192.168.128.236 remote-as 4766

neighbor 192.168.128.236 description # internet-pe7 #

neighbor 192.168.128.236 update-source Loopback0

neighbor 192.168.128.237 remote-as 4766

neighbor 192.168.128.237 description # internet-pe9 #

neighbor 192.168.128.237 update-source Loopback0

neighbor 192.168.128.238 remote-as 4766

neighbor 192.168.128.238 description # internet-pe10 #

neighbor 192.168.128.238 update-source Loopback0

neighbor 192.168.128.244 remote-as 4766

neighbor 192.168.128.244 description # internet-pe6 #

neighbor 192.168.128.244 update-source Loopback0

neighbor 192.168.128.245 remote-as 4766

neighbor 192.168.128.245 description # internet-pe5 #

neighbor 192.168.128.245 update-source Loopback0

neighbor 192.168.128.246 remote-as 4766

neighbor 192.168.128.246 description # internet-pe3 #

neighbor 192.168.128.246 update-source Loopback0

neighbor 192.168.128.247 remote-as 4766

neighbor 192.168.128.247 description # internet-pe4 #

neighbor 192.168.128.247 update-source Loopback0

neighbor 192.168.128.251 remote-as 4766

neighbor 192.168.128.251 description # internet-pe1 #

neighbor 192.168.128.251 update-source Loopback0

neighbor 192.168.128.252 remote-as 4766

neighbor 192.168.128.252 description # internet-pe2 #

neighbor 192.168.128.252 update-source Loopback0

neighbor 192.168.128.254 remote-as 4766

neighbor 192.168.128.254 description # MPLS_GW #

neighbor 192.168.128.254 update-source Loopback0

neighbor 192.168.129.64 remote-as 4766

neighbor 192.168.129.64 description # VPN_수원GSR1 #

neighbor 192.168.129.64 update-source Loopback0

neighbor 192.168.129.65 remote-as 4766

neighbor 192.168.129.65 description # VPN_수원GSR1 #

neighbor 192.168.129.65 update-source Loopback0

!

address-family ipv4

neighbor 192.168.128.66 activate

neighbor 192.168.128.66 soft-reconfiguration inbound

neighbor 192.168.128.3 activate

neighbor 192.168.128.3 soft-reconfiguration inbound

neighbor 192.168.128.4 activate

neighbor 192.168.128.4 soft-reconfiguration inbound

neighbor 192.168.128.235 activate

neighbor 192.168.128.235 soft-reconfiguration inbound

neighbor 192.168.128.236 activate

neighbor 192.168.128.236 soft-reconfiguration inbound

neighbor 192.168.128.237 activate

neighbor 192.168.128.237 soft-reconfiguration inbound

neighbor 192.168.128.238 activate

neighbor 192.168.128.238 soft-reconfiguration inbound

neighbor 192.168.128.244 activate

neighbor 192.168.128.244 soft-reconfiguration inbound

neighbor 192.168.128.245 activate

neighbor 192.168.128.245 soft-reconfiguration inbound

neighbor 192.168.128.246 activate

neighbor 192.168.128.246 soft-reconfiguration inbound

neighbor 192.168.128.247 activate

neighbor 192.168.128.247 soft-reconfiguration inbound

neighbor 192.168.128.251 activate

neighbor 192.168.128.251 soft-reconfiguration inbound

neighbor 192.168.128.252 activate

neighbor 192.168.128.252 soft-reconfiguration inbound

neighbor 192.168.128.254 activate

neighbor 192.168.128.254 soft-reconfiguration inbound

neighbor 192.168.129.64 activate

neighbor 192.168.129.64 send-community both

neighbor 192.168.129.64 soft-reconfiguration inbound

neighbor 192.168.129.65 activate

neighbor 192.168.129.65 send-community both

neighbor 192.168.129.65 soft-reconfiguration inbound

no auto-summary

no synchronization

exit-address-family

!

address-family vpnv

neighbor 192.168.128.66 activate

neighbor 192.168.128.66 send-community extended

neighbor 192.168.128.3 activate

neighbor 192.168.128.3 send-community extended

neighbor 192.168.128.4 activate

neighbor 192.168.128.4 send-community extended

neighbor 192.168.128.235 activate

neighbor 192.168.128.235 send-community extended

neighbor 192.168.128.236 activate

neighbor 192.168.128.236 send-community extended

neighbor 192.168.128.237 activate

neighbor 192.168.128.237 send-community extended

neighbor 192.168.128.238 activate

neighbor 192.168.128.238 send-community extended

neighbor 192.168.128.244 activate

neighbor 192.168.128.244 send-community extended

neighbor 192.168.128.245 activate

neighbor 192.168.128.245 send-community extended

neighbor 192.168.128.246 activate

neighbor 192.168.128.246 send-community extended

neighbor 192.168.128.247 activate

neighbor 192.168.128.247 send-community extended

neighbor 192.168.128.251 activate

neighbor 192.168.128.251 send-community extended

neighbor 192.168.128.252 activate

neighbor 192.168.128.252 send-community extended

neighbor 192.168.128.254 activate

neighbor 192.168.128.254 send-community extended

neighbor 192.168.129.64 activate

neighbor 192.168.129.64 send-community extended

neighbor 192.168.129.65 activate

neighbor 192.168.129.65 send-community extended

exit-address-family

!

address-family ipv4 vrf green_vrf

redistribute connected

redistribute static

no auto-summary

no synchronization

exit-address-family

- ROUTER(config-router)#end

ROUTER#wr

Router#sh ip bgp sum

BGP router identifier 192.168.128.170, local AS number 4766

BGP table version is 10, main routing table version 10

3 network entries and 3 paths using 399 bytes of memory

1 BGP path attribute entries using 60 bytes of memory

1 BGP AS-PATH entries using 24 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP activity 6/15 prefixes, 6/3 paths, scan interval 60 secs

Neighbor V AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.128.171 4 100  13      10       10    0    0 00:06:25        3

위와 같이 neighbor가 잡히면 성공

ROUTER(config)# access-list 1 permit 192.168.128.0 0.0.0.255

ROUTER(config)# access-list 1 permit 192.168.129.0 0.0.0.255

ROUTER(config)# hostname Suwon-VPN-Router

Suwon-VPN-Router (config)# line vty 0 4

Suwon-VPN-Router (config-line)# password vpn

Suwon-VPN-Router (config-line)#exit

Suwon-VPN-Router (config)# enable password vpn

Suwon-VPN-Router (config)#exit